In 2017, the Township submitted a pollutant reduction plan (PRP) with their Municipal Separate Storm Sewer System (MS4) permit renewal package.  The calculations, as prepared by Great Valley Consultants, yielded a total loading of 388,404.11 lbs/year of sediment loading.  For the initial 5-year cycle, a 10% reduction of 38,840.41 lbs/year was required.  Additionally, the required phosphorous reduction was assumed to be met in the process of meeting the sediment reduction credits. The Township tenetaively selected a retrofit of an existing stormwater management BMP in the Green Valley neighborhood of the township.  This single neighborhood/ development constitutes the vast majority of the UA in Lower Heidelberg.  The project was estimated to eliminate the minimum required loading at a cost of $272,500 (in 2017 dollars).  A specific outfall or existing BMP was not selected at the time of PRP preparation.

While administering their MS4 requirements and continuing to plan more details of the BMP implementation, the Township was approached to participate in a joint project with the Wilson School District and the Berks County Conservation District (BCCD).  This project was preliminarily reviewed with the DEP in 2019.  The project would include an application for a Growing Greener Grant with the BCCD and the Wilson School District as the lead entities; the Township agreed to maintain the project for MS4 crediting purposes long-term.  The project consisted of the removal of legacy sediment, restoration of the stream to a more sinuous natural channel instead of a narrow, engineered channel, and removal of invasive species.  Native vegetation was also installed as part of the project.

The project would culminate in approximately 10 acres of restored floodplain and 2500 lineal feet of stream Bank restoration.  To obtain alternative MS4 crediting, the Township agreed to undertake the long-term maintenance of the project area.  As assessed by Land Studies, the designing entity for the project, the stream bank restoration component of the project generated

2,500 LF* 44.88 lbs/LF of treatment

or 112,200 lbs/year of sediment removal as well as a buffer treatment of

1,194.40 lb/acre*50% reduction*1.43 acres = 854 lb per year

This generates a total sediment reduction of 113,054 lb/year or 2.91 times (nearly triple) the initial 5-year requirement predicated in the original approved PRP.  Additionally, by DEP's protocols, the 10% sediment reduction credit automatically meets the phosphorus removal crediting, and as the project greatly exceeded the initial load reductions the phosphorus loading is assumed to be met.  It should be noted that this assumption does not include the hyporheic zone reductions or the floodplain reconnection vegetation treatment as referenced in the DEP’s expert panel on floodplain and stream Bank restoration.  As such, the phosphorus removal crediting is extremely conservative relative to actual conditions.

The Township plans on continuing to retain Land Studies for a period of several years, partially funded by the Growing Greener Grant, with the Township subsequently entering a contract for maintenance.  At a future date, currently projected to be 5-7 years from now, the Township will take over primary maintenance with their own resources with intermittent assessment of the project’s status by Land Studies or another similarly accredited institution.